# BLE Multi-Role

- 🧭 Role Switching (Peripheral ↔ Central ↔ Repeater)

- 📡 Periodic Advertising (PA)

- 🔁 Periodic Sync + Data Exchange

- 📥 MAC-Based Acknowledgment & Sync Filtering

- 🔄 Central selection via "best code"

## ✅ Project Overview: ESP32 BLE Bi-Directional Periodic Advertising Mesh

This project implements a scalable BLE mesh-like architecture using Extended + Periodic Advertising, bi-directional communication, and FreeRTOS task-based design. Devices operate in one of three roles — Central, Repeater, or Peripheral — and communicate without connections using advertising-only PDUs. The goal is to enable:

- 🔄 Data flow from Peripherals to Central

- ✅ ACKs from Central to Repeater and then to Peripheral

- 🔀 Multi-device routing and relay

## 🧭 Core Architecture

### 1. 🔁 Bi-Directional Periodic Advertising

- Central and Repeaters transmit & receive Periodic Advertising PDUs.

- Each device advertises data and listens for sync or ACKs.

- This enables two-way data exchange without BLE connections.

### 2. 📶 Extended + Periodic Advertising (PA)

- Extended ADV (AUX_ADV_IND) used to initiate syncs.

- Periodic ADV (PA) on secondary channels used to send/receive:

  - Sensor Data

  - ACK payloads

  - Role/type/MAC identifiers

### 3. 🔄 Data Flow

PERIPHERAL → REPEATER → CENTRAL   (Sensor data)

CENTRAL → REPEATER → PERIPHERAL   (ACKs)

- Peripherals send periodic packets with sensor data(Dummy Data)

- Repeaters collect, forward to Central.(For now Sending Dummy Data , will add this feature when integrate the sensor code)

- Central generates combined ACK for all synced devices.

- ACK sent once via PA

## 🔀 Role-Based Logic

### 1. 🎭 Role Derivation

- At boot, each device determines its role based on:

  - Wi-Fi connection (enabled/disabled)

  - Previous sync presence

  - Last received ACK

- Roles:

  - Central: Wi-Fi + no upstream ACKs → collects data, sends ACKs

  - Repeater: No Wi-Fi + seen ACKs → bridges data both ways

  - Peripheral: No Wi-Fi + no ACKs → generates sensor data

### 2. 🔀 Dynamic Role Switching

- Device roles can switch at boot or via:

  - Sync timeout

  - Event queues

  - ACK reception/loss

## 📡 Communication Mechanisms

### 1. 📦 Payload Design

Each advertisement contains:

- Type (e.g., A1C, B2R)

- Numeric Code [1-9 ,corresponding to Type

- MAC[3:6]

- Flags for ACK, SYNC, sensor data(Dummy)

### 2. 🧪 MAC Filtering

- Devices only respond to expected MACs

- Ensures correct sync targets and ACKs

### 3. 🔢 SID Calculation

- SID = XOR(mac[0..5]) & 0x0F

- Each advertiser has a unique SID

- Used to manage syncs and match advertisements

## 🔁 Combined ACK Mechanism

- Central receives data from Multiple Repeater / Peripherals

- Builds one ACK with all valid MACs or last 3 bytes

- Sends ACK in Periodic ADV

- Repeaters decode it

### ✅ Actually Use:

- Common tasks and queues/semaphores shared across roles

- Role-specific logic is triggered inside shared handlers and tasks:

  - BLE GAP event handler: gap_event_handler() dispatches to:

    - gap_event_handler_Central()

    - gap_event_handler_Repeater()

    - gap_event_handler_Peripheral()

  - One GAP handler is registered at runtime, switching based on my_current_role.

### ✅ Event-Driven Design:

- xTaskNotify() and xQueueSend() to trigger logic based on:

  - ESP_GAP_BLE_PERIODIC_ADV_REPORT_EVT

  - ESP_GAP_BLE_EXT_ADV_REPORT_EVT

  - Other GAP events

- Synchronization via:

  - SemaphoreHandle_t test_sem

  - QueueHandle_t Scanning_Queue, syncQueue

  - Flags like new_data, sync_stage, is_synced

### ✅ Role Separation:

- Each C file (central.c, repeater.c, peripheral.c) implements:

  - Its own version of GAP event handlers

  - Functions for extended/periodic advertising setup

  - Response to sync/ACK messages

Test Results:

Central:

Repeater :

Peripheral: